ID RFID Reader Writer Instructions

一,Install the connection method

1,Install the driver: PL2303 Driver

2,Check the computer's device manager:[image: image1.png]¥ Prolific USB-to-Serial Comm Port (COM6)




3,Open applet: RFID_APP_EN   following figure:

[image: image2.png]File(F) Card(C) _Help(H)

con bev || _Dise Dev |13 [No devics comneet

& 10H I Kb Out “Enter”
© 08H—10D Keyboard Out
© 06H---10D Write
€ 02H + 04H Read
rOptions
I continuously I incrementally I from file I lock card





4,Click the Conn Dev, as the chart shows that the connection is successful

[image: image3.png]File(F) Card(C) _Help(H)

[ tez]_visc bev | 1ip [Dovice connecied

& 10H I Kb Out “Enter”
© 08H—10D Keyboard Out
© 06H---10D Write
€ 02H + 04H Read
rOptions
I continuously I incrementally I from file I lock card





二：Copy card copy ID card, or write any data to copy card, writing card assistant.

Note: copy the card refers to EM4305 T5577, T5557, etc

1,Copy the card copy ID card operation method: 

First put an ID card Read and Write in the above, click the Read. Remove the ID card, put a Copy card Read and Write in the above, click on the Write. The operation is complete.

2. Write any data to copy card operation method

[image: image4.png]& 10H I Kb Out “Enter”
© 08H—10D Keyboard Out
© 06H---10D Write
coM+oa [ [ Read
rOptions
I continuously I incrementally I from file I lock card





1. To copy the card read and write in the above, select the following one of the input mode.

2. 10 H: refers to the five byte code of ID card, customers can enter 10 hexadecimal data. 

  08-10 h d: refers to the ID card of the 4 byte code, customers can enter 10 decimal data.

  6-10 h d: refers to the ID card after 3 byte code, customers can enter 10 decimal data.

  02 h + 4 h: refers to the ID card weigand data, customer input three and five decimal data

3. Upload data, click the Write, operation is completed.

三：write a card assistant

[image: image5.png]™ continuously I incrementally I from file ™ lock card

"Options





1. Continuously: refers to write the same data to a different card. Choose the project after, point the Write,

2. Incrementally: refers to Write a data, program for the current data automatic 1. Choose the project after, Write,

3. The From file: to guide the data into a text, choose the project after, Write, text format as shown

[image: image6.png]X REE HERO) =80
1111111111
1111111112
1111111113
1111111114
1111111115
1111111116
1111111117





4.After the lock card refers to write data copy card can't write again.

